| Emergency plan example — collision, grounding and/or flooding Reference Material 3.18 |  |
| --- | --- |
| Equipment required and stowage positions |  |
| Life saving appliances | Under seats in saloon and aft deck |
| Grab bag | Wheelhouse just inside door |

|  | Precautionary actions | Crew |
| --- | --- | --- |
|  | All personnel brace for impact. | Master |
|  | Tend to injured. | Crew 1 |
|  | Assess damage to own ship. | Master |
|  | Control ingress of water (if serious, follow flooding procedure). | Crew 2 |
|  | Assist other ship if needed and possible. | Master |
|  | Make radio call. | Master |
|  | Control and report any pollution. | All |
|  | Lay out anchors as required. | Crew 3 |
|  | Owner ship/owner details. | Master |
|  | Flooding |  |
|  | If breach is dangerous raise alarm and position of the flood (bilge alarm may sound), shout: “flood, flood!” | All |
|  | Muster crew at emergency stations and head count. | Crew 1 |
|  | Muster passengers and head count. | Crew 1 |
|  | Rescue any people from the flooded areas. | Crew 1 |
|  | Isolate area and shut all bulkheads watertight doors and hatches. | Crew 2 |
|  | Start pumping or shoring. | Crew 3 |
|  | Begin repairs if possible. | Crew 2 |
|  | Position ship to reduce stress. | Master |
|  | Make radio call as necessary. | Master |
|  | Consider life saving appliance deployment. | Master |
|  | Monitor stability action to correct may be required. | Master |